OpenHarmony系统

传感器-温湿度检测实验案例

指导手册

目录

(一)	实验目的	2

(二)	实验涉及知识点	2

(三)	实验课时	2

(四)	实验内容与要求、最终效果图	2

(五)	详细实验过程操作	3

(六)	扩展内容	5

(七)	源代码获取	5

- 实验目的

- 了解GPIO的基础知识；

- 熟悉修改OpenHarmony中的项目代码和编译脚本的过程；

- 掌握SHT30温湿度传感器的使用方法。

- 实验涉及知识点

- GPIO的基础知识；

- IIC的控制接口；

- SHT30温湿度传感器的使用方法

- 实验课时

1课时。

- 实验内容与要求、最终效果图

- 配置好OpenHarmony南向开发环境；

- 准备好硬件开发板及各种软件工具；

- 在实验箱智慧环境检测模块上利用IIC采集SHT30数据，并控制风扇转速使环境保持合适的温湿度。

最终串口打印如下：

- 详细实验过程操作

1、在device/board/isoftstone/qihang/app目录下新建EBox_IoT_033_SHT30_Env目录；

- 增加wifiiot_pwm.c源文件

- 此文件主要是一些PWM控制接口函数。增加PWM初始化函数PwmInit，初始化PWM的端口，并设置时钟频率。

代码如下：

unsigned int PwmInit(unsigned int clock, unsigned int port) { /*if (hi_pwm_set_clock((hi_pwm_clk_source)clock) != HI_ERR_SUCCESS) { return HI_ERR_FAILURE; } return hi_pwm_init((hi_pwm_port)port);*/ if (hi_pwm_init((hi_pwm_port)port) != HI_ERR_SUCCESS) { return HI_ERR_FAILURE; } return hi_pwm_set_clock((hi_pwm_clk_source)clock); }

- 增加PWM去初始化函数PwmDeinit。

代码如下：

unsigned int PwmDeinit(unsigned int port) { return hi_pwm_deinit((hi_pwm_port)port); }

- 增加PWM驱动函数PwmStart。

代码如下：

unsigned int PwmStart(unsigned int port, unsigned int clock, unsigned short duty, unsigned int freq) { unsigned short hiDuty; unsigned short hiFreq; if ((freq == 0) ¦¦ (duty >= PWM_DUTY_MAX) ¦¦ (duty == PWM_DUTY_MIN)) { printf("%s %s %d HI_ERR_FAILURE \r\n", __FILE__, __FUNCTION__, __LINE__); return HI_ERR_FAILURE; } if ((clock / freq) > PWM_SHORT_MAX) { printf("%s %s %d HI_ERR_FAILURE \r\n", __FILE__, __FUNCTION__, __LINE__); return HI_ERR_FAILURE; } hiFreq = (unsigned short)(clock / freq); hiDuty = (duty * hiFreq) / PWM_DUTY_MAX; return hi_pwm_start((hi_pwm_port)port, hiDuty, hiFreq); }

- 增加PWM停止函数PwmStop。

代码如下：

unsigned int PwmStop(unsigned int port) { return hi_pwm_stop((hi_pwm_port)port); }

- 3、增加wifiiot_pwm.h头文件

- 此文件主要是一些枚举定义，以及wifiiot_pwm.c源文件中函数的声明。

代码如下：

typedef enum { /** 160 MHz working clock */ WIFI_IOT_PWM_CLK_160M, /** 24 MHz or 40 MHz external crystal */ WIFI_IOT_PWM_CLK_XTAL, /** Maximum value */ WIFI_IOT_PWM_CLK_MAX } WifiIotPwmClkSource; typedef enum { WIFI_IOT_PWM_PORT_PWM0 = 0, WIFI_IOT_PWM_PORT_PWM1 = 1, WIFI_IOT_PWM_PORT_PWM2 = 2, WIFI_IOT_PWM_PORT_PWM3 = 3, WIFI_IOT_PWM_PORT_PWM4 = 4, WIFI_IOT_PWM_PORT_PWM5 = 5, WIFI_IOT_PWM_PORT_MAX } WifiIotPwmPort; unsigned int PwmInit(unsigned int clock, unsigned int port); unsigned int PwmDeinit(unsigned int port); unsigned int PwmStart(unsigned int port, unsigned int clock, unsigned short duty, unsigned int freq); unsigned int PwmStop(unsigned int port);

- 4、增加ebox_env_motor.c源文件

- 此文件主要是PWM控制风扇功能。增加风扇初始化函数EnvMotorInit，风扇连接的是GPIO2引脚，将GPIO2复用为PWM功能，对应的是PWM2，对PWM2进行初始化。

代码如下：

void EnvMotorInit(void) { /* 电机接在GPIO-02引脚，初始化GPIO-02引脚 */ u32 ret = IoTGpioInit(MOTOR_IO_GPIO_02); if(ret != KP_ERR_SUCCESS){ printf("IoTGpioInit failed :%#x \r\n",ret); return; } /* 设置复用为PWM功能 */ ret = IoTGpioSetFunc(MOTOR_IO_GPIO_02, IOT_GPIO_FUNC_GPIO_2_PWM2_OUT); if(ret != KP_ERR_SUCCESS){ printf("IoTGpioSetFunc failed :%#x \r\n",ret); return; } /* PWM初始化 */ ret = PwmDeinit(MOTOR_PWM_PORT_02); if(ret != KP_ERR_SUCCESS){ printf("PwmDeinit failed :%#x \r\n",ret); return; } ret = PwmInit(MOTOR_PWM_CLOCK_TYPE, MOTOR_PWM_PORT_02); if(ret != KP_ERR_SUCCESS){ printf("PwmInit failed :%#x \r\n",ret); return; } printf("----- EnvMotorInit success! -----\r\n"); }

- 增加风扇控制函数EnvMotorCtrl。

代码如下：

void EnvMotorCtrl(motor_speed_level state) { /* 初始化变量 */ u32 ret = 0; switch (state) { case MOTOR_SPEED_LEVEL_0: /* 关闭电机 */ ret = PwmStop(MOTOR_PWM_PORT_02); if (ret != KP_ERR_SUCCESS) { printf("PwmStop failed ret : %#x \r\n", ret); return; } printf("stop motor \r\n"); return; case MOTOR_SPEED_LEVEL_1: /* 一档风扇 */ //ret = PwmStart(MOTOR_PWM_PORT_02, MOTOR_PWM_CLOCK_160M, PWM_DUTY_MAX * MOTOR_SPEED_LEVEL_1 / MOTOR_SPEED_LEVEL_MAX, freq); ret = PwmStart(MOTOR_PWM_PORT_02, MOTOR_PWM_CLOCK_160M, 25, freq); if (ret != KP_ERR_SUCCESS) { printf("PwmStart failed ret : %#x \r\n", ret); return; } printf("start motor level 1 \r\n"); return; case MOTOR_SPEED_LEVEL_2: /* 二档风扇 */ //ret = PwmStart(MOTOR_PWM_PORT_02, MOTOR_PWM_CLOCK_160M, PWM_DUTY_MAX * MOTOR_SPEED_LEVEL_2 / MOTOR_SPEED_LEVEL_MAX, freq); ret = PwmStart(MOTOR_PWM_PORT_02, MOTOR_PWM_CLOCK_160M, 50, freq); if (ret != KP_ERR_SUCCESS) { printf("PwmStart failed ret : %#x \r\n", ret); return; } printf("start motor level 2 \r\n"); return; case MOTOR_SPEED_LEVEL_3: /* 三档风扇 */ //ret = PwmStart(MOTOR_PWM_PORT_02, MOTOR_PWM_CLOCK_160M, PWM_DUTY_MAX * MOTOR_SPEED_LEVEL_3 / MOTOR_SPEED_LEVEL_MAX, freq); ret = PwmStart(MOTOR_PWM_PORT_02, MOTOR_PWM_CLOCK_160M, 75, freq); if (ret != KP_ERR_SUCCESS) { printf("PwmStart failed ret : %#x \r\n", ret); return; } printf("start motor level 3 \r\n"); return; default: /* 无效风扇转速 */ printf("invalid motor level \r\n"); return; } }

- 5、增加ebox_env_motor.h头文件

- 此文件主要是风扇转速宏定义，以及ebox_env_motor.c源文件中函数的声明。

代码如下：

#ifndef __EBOX_ENV_MOTOR_H__ #define __EBOX_ENV_MOTOR_H__ typedef enum { MOTOR_SPEED_LEVEL_0 = 0, /* stop */ MOTOR_SPEED_LEVEL_1, MOTOR_SPEED_LEVEL_2, MOTOR_SPEED_LEVEL_3, MOTOR_SPEED_LEVEL_MAX /* invalid */ } motor_speed_level; /* 初始化风扇 */ void AgricultureMotorInit(void); /* 风扇控制转动 */ void AgricultureMotorCtrl(motor_speed_level state); #endif

- 6、增加ebox_env_sht30.c源文件

- 此文件主要是SHT30模块的处理函数。增加端口及常量的定义及SHT30模块初始化函数，智慧环境检测模块上的SHT连接的是GPIO0和GPIO1两个引脚，需要将引脚复用为IIC1功能，并对IIC1进行初始化。

代码如下：

#define SHT30_SDA_IO_GPIO_00 0 #define SHT30_SCL_IO_GPIO_01 1 #define SHT30_IIC_IDX_1 1 #define SHT30_IIC_BAUDRATE 400000 /* IIC功能初始化 */ void ShtIICInit(void) { // 初始化SHT30 SDA引脚 u32 ret = IoTGpioInit(SHT30_SDA_IO_GPIO_00); if(ret != 0){ printf("IoTGpioInit failed :%#x \r\n",ret); return; } /* 设置复用为IIC功能 */ ret = IoTGpioSetFunc(SHT30_SDA_IO_GPIO_00, IOT_GPIO_FUNC_GPIO_0_I2C1_SDA); if(ret != 0){ printf("IoTGpioSetFunc failed :%#x \r\n",ret); return; } // 初始化SHT30 SCL引脚 ret = IoTGpioInit(SHT30_SCL_IO_GPIO_01); if(ret != 0){ printf("IoTGpioInit failed :%#x \r\n",ret); return; } /* 设置复用为IIC功能 */ ret = IoTGpioSetFunc(SHT30_SCL_IO_GPIO_01, IOT_GPIO_FUNC_GPIO_1_I2C1_SCL); if(ret != 0){ printf("IoTGpioSetFunc failed :%#x \r\n",ret); return; } /* IIC初始化 */ ret = IoTI2cInit(SHT30_IIC_IDX_1, SHT30_IIC_BAUDRATE); if (ret != 0) { printf("IoTI2cInit failed :%#x \r\n",ret); return; } /* IIC设置波特率 */ ret = IoTI2cSetBaudrate(SHT30_IIC_IDX_1, SHT30_IIC_BAUDRATE); if (ret != 0) { printf("IoTI2cSetBaudrate failed :%#x \r\n",ret); return; } printf("----- ShtIICInit success! -----\r\n"); }

- 增加SHT模块计算CRC校验和的函数SHT3XCalcCrc。

代码如下：

static unsigned char SHT3XCalcCrc(unsigned char data[], unsigned int nbrOfBytes) { unsigned char bit; // bit mask unsigned char crc = 0xFF; // calculated checksum unsigned char byteCtr; // byte counter // calculates 8-Bit checksum with given polynomial for(byteCtr = 0; byteCtr < nbrOfBytes; byteCtr++) { crc ^= (data[byteCtr]); for(bit = 8; bit > 0; --bit) { if(crc & 0x80) crc = (crc << 1) ^ POLYNOMIAL; else crc = (crc << 1); } } return crc; }

- 增加MPU6050模块校验CRC的函数SHT3XCheckCrc。

代码如下：

static etError SHT3XCheckCrc(unsigned char data[], unsigned int nbrOfBytes, unsigned char checksum) { unsigned char crc; // calculated checksum // calculates 8-Bit checksum crc = SHT3XCalcCrc(data, nbrOfBytes); // verify checksum if(crc != checksum) return CHECKSUM_ERROR; else return NO_ERROR; }

- 增加SHT30模块通过I2C读取串行数据的函数SHT3xReadSerialNumber。

代码如下：

static int SHT3xReadSerialNumber(unsigned int *serialNumber) { unsigned int ret = 0; unsigned char serialNumWords[6] = {0}; unsigned char cmd[2] = {0x37,0x80,}; ret = IoTI2cWrite(SHT30_IIC_IDX_1, ((unsigned char)0x44) << 1, cmd, sizeof(cmd)); if(ret != 0){ printf("SHT3xReadSerialNumber IoTI2cWrite failed ret :%#x \r\n",ret); return ret; } ret = IoTI2cRead(SHT30_IIC_IDX_1, ((unsigned char)0x44) << 1 ¦ 0x01, serialNumWords, sizeof(serialNumWords)); if(ret != 0){ printf("SHT3xReadSerialNumber IoTI2cRead failed ret :%#x \r\n",ret); return ret; } //dump_buf(serialNumWords,sizeof(serialNumWords)); ret = SHT3XCheckCrc(serialNumWords,2,serialNumWords[2]); if(ret != NO_ERROR){ printf("read serial number crc check failed \r\n"); return ret; } ret = SHT3XCheckCrc(&serialNumWords[3],2,serialNumWords[5]); if(ret != NO_ERROR){ printf("read serial number crc check failed \r\n"); return ret; } return ret; }

- 增加SHT30模块写命令的函数SHT3xWriteCMD。

代码如下：

static int SHT3xWriteCMD(unsigned short cmd) { unsigned int ret = 0; unsigned char sendbuf[2] = {0}; unsigned char rcvbuf[2] = {0}; sendbuf[0] = (cmd & 0xff00) >> 8; sendbuf[1] = cmd & 0xff; //dump_buf(sendbuf,2); ret = IoTI2cWrite(SHT30_IIC_IDX_1, ((unsigned char)0x44) << 1, sendbuf, sizeof(sendbuf)); if(ret != 0){ printf("SHT3x_WriteCMD IoTI2cWrite failed, cmd : %#x, ret :%#x \r\n", cmd, ret); return ret; } return 0; }

- 增加SHT30模块读取4字节数据并进行CRC校验的函数SHT3xRead4BytesDataAndCrc。

代码如下：

static int SHT3xRead4BytesDataAndCrc(unsigned short *data) { unsigned int ret = 0; unsigned char sendbuf[2] = {0}; unsigned char rcvbuf[6] = {0}; if(data == NULL){ printf("SHT3xRead4BytesDataAndCrc invalid para \r\n"); return ret; } ret = IoTI2cRead(SHT30_IIC_IDX_1, ((unsigned char)0x44) << 1 ¦ 0x01, rcvbuf, sizeof(rcvbuf)); if(ret != 0){ printf("SHT3xRead4BytesDataAndCrc IoTI2cRead failed ret :%#x \r\n",ret); return ret; } ret = SHT3XCheckCrc(rcvbuf,2,rcvbuf[2]); if(ret != NO_ERROR){ printf("read serial number crc check failed \r\n"); return ret; } ret = SHT3XCheckCrc(&rcvbuf[3],2,rcvbuf[5]); if(ret != NO_ERROR){ printf("read serial number crc check failed \r\n"); return ret; } data[0] = rcvbuf[0] << 8 ¦ rcvbuf[1]; data[1] = rcvbuf[3] << 8 ¦ rcvbuf[4]; return 0; }

- 增加SHT30模块读取2字节数据并进行CRC校验的函数SHT3xRead2BytesDataAndCrc。

代码如下：

static int SHT3xRead2BytesDataAndCrc(unsigned short *data) { unsigned int ret = -1; unsigned char sendbuf[2] = {0}; unsigned char rcvbuf[3] = {0}; if(data == NULL){ printf("SHT3xRead2BytesDataAndCrc invalid para \r\n"); return ret; } ret = IoTI2cRead(SHT30_IIC_IDX_1, ((unsigned char)0x44) << 1 ¦ 0x01, rcvbuf, sizeof(rcvbuf)); if(ret != 0){ printf("SHT3xRead2BytesDataAndCrc IoTI2cRead failed ret :%#x \r\n",ret); return ret; } ret = SHT3XCheckCrc(rcvbuf,2,rcvbuf[2]); if(ret != NO_ERROR){ printf("read serial number crc check failed \r\n"); return ret; } data[0] = rcvbuf[0] << 8 ¦ rcvbuf[1]; return 0; }

- 增加SHT30模块复位的函数SHT3XSoftReset。

代码如下：

void SHT3XSoftReset(void) { SHT3xWriteCMD(CMD_SOFT_RESET); }

- 增加SHT30模块获取模块状态的函数SHT3XReadStatus。

代码如下：

static void SHT3XReadStatus(unsigned short* status) { SHT3xWriteCMD(CMD_READ_STATUS); SHT3xRead2BytesDataAndCrc(status); }

- 增加SHT30模块清除所有告警的函数SHT3XClearAllAlertFlags。

代码如下：

static void SHT3XClearAllAlertFlags(void) { SHT3xWriteCMD(CMD_CLEAR_STATUS); }

- 增加SHT30模块根据采集到的数据计算温度的函数SHT3XCalcTemperature。

代码如下：

static float SHT3XCalcTemperature(unsigned short rawValue) { return 175.0f * (float)rawValue / 65535.0f - 45.0f; }

- 增加SHT30模块根据采集到的数据计算湿度的函数SHT3XCalcHumidity。

代码如下：

static float SHT3XCalcHumidity(unsigned short rawValue) { return 100.0f * (float)rawValue / 65535.0f; }

- 增加SHT30模块初始化的函数SHT3XInit。

代码如下：

void SHT3XInit(void) { int ret = 0; unsigned short data[2] = {0}; SHT3xWriteCMD(CMD_READ_SERIALNBR); osDelay(100); SHT3xWriteCMD(CMD_MEAS_PERI_2_M); }

- 增加利用IIC从SHT30设备读取温度和湿度数据的函数SHT3XReadMeasurementBuffer。

代码如下：

void SHT3XReadMeasurementBuffer(float* temperature, float* humidity) { unsigned int rawValueTemp = 0; SHT3xWriteCMD(CMD_FETCH_DATA); SHT3xRead4BytesDataAndCrc((unsigned short *)&rawValueTemp); //dump_buf((unsigned char *)&rawValueTemp,sizeof(rawValueTemp)); *temperature = SHT3XCalcTemperature(rawValueTemp); *humidity = SHT3XCalcHumidity(*((unsigned short *)(&rawValueTemp)+1)); }

- 7、增加ebox_env_sht30.h头文件

- 此文件主要是SHT30模块的相关端口及命令定义，以及ebox_env_sht30.c源文件中部分接口函数的声明。

代码如下：

#ifndef __EBOX_ENV_SHT30_H__ #define __EBOX_ENV_SHT30_H__ #define I2C_REG_ARRAY_LEN 64 #define SHT3X_DEV_ADDR 0x80 /* 11000 0 */ #define ES8311_REG_ADDR 0x10 #define I2C_SEND_LEN_2 2 #define I2C_RECV_LEN_1 1 #define POLYNOMIAL 0x131 // P(x) = x^8 + x^5 + x^4 + 1 = 100110001 typedef enum{ NO_ERROR = 0x00, // no error ACK_ERROR = 0x01, // no acknowledgment error CHECKSUM_ERROR = 0x02, // checksum mismatch error TIMEOUT_ERROR = 0x04, // timeout error PARM_ERROR = 0x80, // parameter out of range error }etError; typedef enum{ CMD_READ_SERIALNBR = 0x3780, // read serial number CMD_READ_STATUS = 0xF32D, // read status register CMD_CLEAR_STATUS = 0x3041, // clear status register CMD_HEATER_ENABLE = 0x306D, // enabled heater CMD_HEATER_DISABLE = 0x3066, // disable heater CMD_SOFT_RESET = 0x30A2, // soft reset CMD_MEAS_CLOCKSTR_H = 0x2C06, // measurement: clock stretching, high repeatability CMD_MEAS_CLOCKSTR_M = 0x2C0D, // measurement: clock stretching, medium repeatability CMD_MEAS_CLOCKSTR_L = 0x2C10, // measurement: clock stretching, low repeatability CMD_MEAS_POLLING_H = 0x2400, // measurement: polling, high repeatability CMD_MEAS_POLLING_M = 0x240B, // measurement: polling, medium repeatability CMD_MEAS_POLLING_L = 0x2416, // measurement: polling, low repeatability CMD_MEAS_PERI_05_H = 0x2032, // measurement: periodic 0.5 mps, high repeatability CMD_MEAS_PERI_05_M = 0x2024, // measurement: periodic 0.5 mps, medium repeatability CMD_MEAS_PERI_05_L = 0x202F, // measurement: periodic 0.5 mps, low repeatability CMD_MEAS_PERI_1_H = 0x2130, // measurement: periodic 1 mps, high repeatability CMD_MEAS_PERI_1_M = 0x2126, // measurement: periodic 1 mps, medium repeatability CMD_MEAS_PERI_1_L = 0x212D, // measurement: periodic 1 mps, low repeatability CMD_MEAS_PERI_2_H = 0x2236, // measurement: periodic 2 mps, high repeatability CMD_MEAS_PERI_2_M = 0x2220, // measurement: periodic 2 mps, medium repeatability CMD_MEAS_PERI_2_L = 0x222B, // measurement: periodic 2 mps, low repeatability CMD_MEAS_PERI_4_H = 0x2334, // measurement: periodic 4 mps, high repeatability CMD_MEAS_PERI_4_M = 0x2322, // measurement: periodic 4 mps, medium repeatability CMD_MEAS_PERI_4_L = 0x2329, // measurement: periodic 4 mps, low repeatability CMD_MEAS_PERI_10_H = 0x2737, // measurement: periodic 10 mps, high repeatability CMD_MEAS_PERI_10_M = 0x2721, // measurement: periodic 10 mps, medium repeatability CMD_MEAS_PERI_10_L = 0x272A, // measurement: periodic 10 mps, low repeatability CMD_FETCH_DATA = 0xE000, // readout measurements for periodic mode CMD_R_AL_LIM_LS = 0xE102, // read alert limits, low set CMD_R_AL_LIM_LC = 0xE109, // read alert limits, low clear CMD_R_AL_LIM_HS = 0xE11F, // read alert limits, high set CMD_R_AL_LIM_HC = 0xE114, // read alert limits, high clear CMD_W_AL_LIM_HS = 0x611D, // write alert limits, high set CMD_W_AL_LIM_HC = 0x6116, // write alert limits, high clear CMD_W_AL_LIM_LC = 0x610B, // write alert limits, low clear CMD_W_AL_LIM_LS = 0x6100, // write alert limits, low set CMD_NO_SLEEP = 0x303E, }etCommands; #define LITTLE_ENDIAN typedef union { unsigned short u16; struct{ #ifdef LITTLE_ENDIAN // bit-order is little endian unsigned short CrcStatus : 1; // write data checksum status unsigned short CmdStatus : 1; // command status unsigned short Reserve0 : 2; // reserved unsigned short ResetDetected : 1; // system reset detected unsigned short Reserve1 : 5; // reserved unsigned short T_Alert : 1; // temperature tracking alert unsigned short RH_Alert : 1; // humidity tracking alert unsigned short Reserve2 : 1; // reserved unsigned short HeaterStatus : 1; // heater status unsigned short Reserve3 : 1; // reserved unsigned short AlertPending : 1; // alert pending status #else // bit-order is big endian unsigned short AlertPending : 1; unsigned short Reserve3 : 1; unsigned short HeaterStatus : 1; unsigned short Reserve2 : 1; unsigned short RH_Alert : 1; unsigned short T_Alert : 1; unsigned short Reserve1 : 5; unsigned short ResetDetected : 1; unsigned short Reserve0 : 2; unsigned short CmdStatus : 1; unsigned short CrcStatus : 1; #endif }bit; } regStatus; void ShtIICInit(void); void SHT3XSoftReset(void); void SHT3XInit(void); static int SHT3xReadSerialNumber(unsigned int *serialNumber); void SHT3XReadMeasurementBuffer(float* temperature, float* humidity); #endif

- 8、增加ebox_env_module.c源文件

- 增加智慧环境检测模块温湿度检测任务函数EnvShtTask，通过定时查询SHT30模块采集的温度和湿度数据来确定环境状况，如果湿度过大则调用风扇控制函数启动风扇来保持环境湿度。

代码如下：

void EnvShtTask(void) { ShtIICInit(); //osDelay(TASK_DELAY); SHT3XInit(); osDelay(TASK_DELAY); /* 初始化变量 */ float temperature = 0.0; float humidity = 0.0; static motor_speed_level motorSpeed = MOTOR_SPEED_LEVEL_0; while (1) { /* 查询温度和湿度数据 */ SHT3XReadMeasurementBuffer(&temperature, &humidity); //hi_watchdog_feed(); printf("temperature:%.2f, humidity:%.2f \r\n", temperature, humidity); if (humidity > 80.0) { if (motorSpeed != MOTOR_SPEED_LEVEL_3) { EnvMotorCtrl(MOTOR_SPEED_LEVEL_3); motorSpeed = MOTOR_SPEED_LEVEL_3; printf("adjust motor speed to level 3 \r\n"); } } else if (humidity > 70.0) { if (motorSpeed != MOTOR_SPEED_LEVEL_2) { EnvMotorCtrl(MOTOR_SPEED_LEVEL_2); motorSpeed = MOTOR_SPEED_LEVEL_2; printf("adjust motor speed to level 2 \r\n"); } } else if (humidity > 60.0) { if (motorSpeed != MOTOR_SPEED_LEVEL_1) { EnvMotorCtrl(MOTOR_SPEED_LEVEL_1); motorSpeed = MOTOR_SPEED_LEVEL_1; printf("adjust motor speed to level 1 \r\n"); } } else { if (motorSpeed != MOTOR_SPEED_LEVEL_0) { EnvMotorCtrl(MOTOR_SPEED_LEVEL_0); motorSpeed = MOTOR_SPEED_LEVEL_0; printf("adjust motor speed to level 0 \r\n"); } } osDelay(TASK_DELAY); } return; }

- 增加主任务函数，创建智慧环境检测任务。

代码如下：

void EnvSht30Sample(void) { /* 各模块初始化 */ //EnvLedInit(); EnvMotorInit(); osDelay(TASK_DELAY); /* 设置线程属性 */ osThreadAttr_t attr; attr.name = "EnvShtTask"; attr.attr_bits = 0U; attr.cb_mem = NULL; attr.cb_size = 0U; attr.stack_mem = NULL; attr.stack_size = 1024 * 4; attr.priority = 20; /* 创建温湿度处理线程 */ attr.name = "EnvShtTask"; osThreadId_t threadIDSht = osThreadNew((osThreadFunc_t)EnvShtTask, NULL, &attr); if (threadIDSht == NULL) { printf("Falied to create EnvShtTask!\r\n"); } }

9、增加BUILD.gn文件

内容如下：

static_library("Env_module_sample") { sources = [ "ebox_env_module.c", "ebox_env_motor.c", "ebox_env_sht30.c", "wifiiot_pwm.c", ] include_dirs = [ "//base/iot_hardware/peripheral/interfaces/kits", "../../iot_hardware_hals/include", ] }

10、编辑device/board/isoftstone/qihang/app/BUILD.gn文件

代码如下：

import("//build/lite/config/component/lite_component.gni") lite_component("app") { features = [ "EBox_IoT_033_SHT30_Env:Env_module_sample", ] }

11、编译并烧录到实验箱智慧环境检测模块上，按下复位键启动，当向SHT芯片呵气使得SHT芯片的环境湿度超过阈值时，风扇转动降低湿度，当环境湿度恢复正常时，风扇停止转动，串口打印如下

- 扩展内容

无