适用于博赛开发板

LED实验

项目名称：LEDtest  （源码根据自己的命名变化）

主文件夹下的BUILD.gn

group("LEDtest")

{

deps=["SOURCE_LED:LEDtest",

"//device/bossay/hi3861_l0/sdk_liteos:wifiiot_sdk",

"../common/iot_wifi:iot_wifi",

]

}

主文件夹下config.json

{

"product_name": "LEDtest",

"ohos_version": "OpenHarmony 3.0",

"device_company": "bossay",

"board": "hi3861_l0",

"kernel_type": "liteos_m",

"kernel_version": "",

"subsystems":

[

{

"subsystem": "iot_hardware",

"components":

[

{ "component": "iot_controller", "features":[] }

]

},

{

"subsystem": "distributed_schedule",

"components":

[

{ "component": "samgr_lite", "features":[] }

]

},

{

"subsystem": "security",

"components":

[

{ "component": "hichainsdk", "features":[] },

{ "component": "deviceauth_lite", "features":[] },

{ "component": "huks", "features":

[

"huks_config_file=\"hks_config_lite.h\"",

"huks_mbedtls_path=\"//device/bossay/hi3861_l0/sdk_liteos/third_party/mbedtls/include/\""

]

}

]

},

{

"subsystem": "startup",

"components":

[

{ "component": "bootstrap_lite", "features":[] },

{ "component": "syspara_lite", "features":

[

"enable_ohos_startup_syspara_lite_use_thirdparty_mbedtls=false"

]

}

]

},

{

"subsystem": "utils",

"components":

[

{ "component": "file", "features":[] },

{ "component": "kv_store", "features":[] },

{ "component": "os_dump", "features":[]}

]

}

],

"third_party_dir": "//device/bossay/hi3861_l0/sdk_liteos/third_party",

"product_adapter_dir":"//vendor/bossay/hi3861_l0/hals"

}

SOURCE_LEDtest 下的BUILD.gn

static_library("LEDtest")

{

sources=["LEDtest.c"]

include_dirs=[

"//utils/native/lite/include",

"//base/iot_hardware/peripheral/interfaces/kits",

"//device/bossay/hi3861_l0/iot_hardware_hals/include",

"//device/bossay/hi3861_l0/sdk_liteos/include"

]

}

注意文件中的hi3861_l0是小写字母l和数字0：

LEDtest.c

#include<stdio.h>

#include"ohos_init.h"

#include"iot_gpio_ex.h"

#include"iot_gpio.h"

#define OUT_GPIO13 13

static void LED(void)

{

IoTGpioInit(OUT_GPIO13);

IoTGpioSetFunc(OUT_GPIO13,IOT_GPIO_FUNC_GPIO_13_GPIO);

IoTGpioSetDir(OUT_GPIO13,IOT_GPIO_DIR_OUT);

IoTGpioSetOutputVal(OUT_GPIO13,1);

}

APP_FEATURE_INIT(LED);

闪烁的LED实验

项目名称：FLASHING

主文件夹下的BUILD.gn

group("FLASHING")

{

deps=["SOURCE_FLASHING: FLASHING",

"//device/bossay/hi3861_l0/sdk_liteos:wifiiot_sdk",

"../common/iot_wifi:iot_wifi",

]

}

主文件夹下config.json，与LED实验基本一致，只是项目名称有区别。

{

"product_name": "FLASHING",

"ohos_version": "OpenHarmony 3.0",

"device_company": "bossay",

"board": "hi3861_l0",

"kernel_type": "liteos_m",

"kernel_version": "",

"subsystems":

[

{

"subsystem": "iot_hardware",

"components":

[

{ "component": "iot_controller", "features":[] }

]

},

{

"subsystem": "distributed_schedule",

"components":

[

{ "component": "samgr_lite", "features":[] }

]

},

{

"subsystem": "security",

"components":

[

{ "component": "hichainsdk", "features":[] },

{ "component": "deviceauth_lite", "features":[] },

{ "component": "huks", "features":

[

"huks_config_file=\"hks_config_lite.h\"",

"huks_mbedtls_path=\"//device/bossay/hi3861_l0/sdk_liteos/third_party/mbedtls/include/\""

]

}

]

},

{

"subsystem": "startup",

"components":

[

{ "component": "bootstrap_lite", "features":[] },

{ "component": "syspara_lite", "features":

[

"enable_ohos_startup_syspara_lite_use_thirdparty_mbedtls=false"

]

}

]

},

{

"subsystem": "utils",

"components":

[

{ "component": "file", "features":[] },

{ "component": "kv_store", "features":[] },

{ "component": "os_dump", "features":[]}

]

}

],

"third_party_dir": "//device/bossay/hi3861_l0/sdk_liteos/third_party",

"product_adapter_dir":"//vendor/bossay/hi3861_l0/hals"

}

SOURCE_FLASHING文件夹下的BUILD.gn

static_library("FLASHING")

{

sources=["FLASHING.c"]

include_dirs=[

"//utils/native/lite/include",

"//base/iot_hardware/peripheral/interfaces/kits",

"//device/bossay/hi3861_l0/iot_hardware_hals/include",

"//device/bossay/hi3861_l0/sdk_liteos/include"

]

}

SOURCE_FLASHING文件夹下的FLASHING.c

#include<stdio.h>

#include"ohos_init.h"

#include"iot_gpio_ex.h"

#include"iot_gpio.h"

#define LED_GPIO 9

static void led(void* args)

{

printf(“led running…”);

IoTGpioInit(LED_GPIO);

IoTGpioSetDir(LED_GPIO,IOT_GPIO_DIR_OUT);

IoTGpioSetFunc(LED_GPIO,IOT_GPIO_FUNC_GPIO_9_GPIO);

int v=1;

while(1)

{

IoTGpioSetOutputVal(LED_GPIO,v);

v=1-v;

usleep(200*1000)

}

}

APP_FEATURE_INIT(led);

呼吸灯实验  BREATHE.c

#include<stdio.h>

#include"ohos_init.h"

#include"iot_gpio_ex.h"

#include"iot_gpio.h"

#include "iot_pwm.h"

#define PWM_GPIO 9

void pwm_entry()

{

printf("pwm_entry called \n");

IoTGpioInit(PWM_GPIO);

IoTGpioSetDir(PWM_GPIO,IOT_GPIO_DIR_OUT);

IoTGpioSetFunc(PWM_GPIO,IOT_GPIO_FUNC_GPIO_9_PWM0_OUT);

IoTPwmInit(0);

int i;

while(1)

{

for(i=0;i<100;i++)

{IoTPwmStart(0,i,40000);

usleep(10*1000);

}

for(i=100;i>=0;i--)

{IoTPwmStart(0,i,40000);

usleep(10*1000);

}

}

}

APP_FEATURE_INIT(pwm_entry);

跑马灯实验 MARQUEE.c

#include<stdio.h>

#include"ohos_init.h"

#include"iot_gpio_ex.h"

#include"iot_gpio.h"

void led(void *args)

{ int i;

printf("led thread running...");

for(i=9;i<=13;i++)

{IoTGpioInit(i);

IoTGpioSetDir(i,IOT_GPIO_DIR_OUT);

IoTGpioSetFunc(i,0);

}

IoTGpioSetFunc(13,IOT_GPIO_FUNC_GPIO_13_GPIO);

int c=9;

while(1)

{

for(i=9;i<=13;i++)

{

if (i==9)

{

IoTGpioSetOutputVal(13,0);

}else

{

IoTGpioSetOutputVal(i-1,0);

}

IoTGpioSetOutputVal(i,1);

usleep(200*1000);

}

}

}

APP_FEATURE_INIT(led);